SmartFlow: Prioritize Emergency Vehicle and Traffic Rules Monitoring System

SmartFlow

Step1: Start

Step2: Set up RSTP connection

Step3: If failed to read frame from RSTP stream go to step 11 else go to step 5

Step4: Initialize the data base connection

Step5: While true repeat steps from 6 to

Step6: Display live transmission from RSTP stream

Step7: Capture the frames from the RSTP stream at regular interval of time and save to database

Step8: If green delay is > 5 seconds go to step 7 else go to step 9

Step7: Call function EmergencyVehicle()

Step8: If emergency vehicle detected goto step12 else go to step 9

Step9: Call function DelayCalculation(img)

Step10: Update the green signal duration returned by DelayCalulation ()

Step11: Turn on Yellow Led indicating failed to read from RSTP stream and go to step 14

Step12: Turn on red led and update the green signal

Step13: If keyboard interruption go to step 12 else go to step6

Step14: Stop

Module 1: Congestion control

DelayCalculation(img)

Step1: Load VehicleDetector()

Step2: Initialize the source image and destination image path and speed

Step3: Read the image

Step4: Draw the boxes to vehicles

Step5: count the number of vehicles

Step6: Display the number of vehicles

Step7: Save the image with bounding boxes

Step8: Calculate the green delay

GreenDelay = no. of vehicles * speed

Step9: Return speed

VehicleDetector

Step1: Load YOLO4 model weights and configuration files using OpenCV

Step2: Initialize the detection model using cv2.dn DetectionModel() and set input parameters, such as

Size and scale

Step3: Specify the classes that correspond to vehicles in the COCO dataset

Step4: detect_vehicles(img)

Step1: Read the image

Step2: Detect the object in the image using YOLO4 model

Step3: Store the bounding boxes of the detected vehicles in a list

Step4: Return the list of boxes representing the detected vehicles.

Module 2: emergency Vehicle detector

EmergencyVehicle

Step1: Set the path to the Tesseract OCR engine

Step2: Load the pre-trained EAST text detection model (frozen_east_text_detection.pb) using OpenCV's DNN module.

Step3: Read the image using from data base

Step4: Resize the input frame to a fixed size to match the input size expected by the EAST model.

Step5: Convert the resized frame to a blob suitable for input to the neural network (cv2.dnn.blobFromImage()).

Step6: Pass the blob through the EAST model (net.forward()) to obtain predictions draw boxes.

Step7: Text Recognition (OCR) Extract the region of interest (ROI) from the original frame. Apply Tesseract OCR (pytesseract.image_to_string()) to recognize text within the ROI.

Step8: Return true if text is ‘AMBULANCE’ else false

Architecture diagram

ESP32-Cam

Real Time Traffic

Emergency vehicle

Arduino uno

Capture video

Capture image

Programing ESP32

Arduino Mega

Traffic Signal

Update the signal

Database

System

Save to Database

Emergency vehicle

Detected or

Load Green Delay to Arduino

Convert video to frames

Save or Extract image

Figure 1: Architecture diagram

Above figure 1 represents the complete architecture of the SmartFlow system. The Arduino uno is used to program the ESP32 camera. Camera then captures the image of real traffic in one lane and it is stored in the database. Then that image is loaded to the YOLO algorithm to count the number of vehicles in the image or in a lane and based on the number of vehicles green delay is calculated by using formula speed*number of vehicles hear speed is the average speed required by the vehicles to pass through signal. Using Arduino Mega traffic lights are controlled.

Data flow diagram

Emergency vehicle detection

Capture Image

Send to server to detect vehicle density

Updating the signal

Calculating the green signal Dealy

Figure 2: Data flow diagram

As shown in the above figure 2, ESP32-camera captures the image and store it in the server, then YOLO algorithm processes the image and detect the congestion and calculate the green signal timing whit help of formula average speed * number of vehicles, according that calculation the traffic signal is controlled. If there is any emergency vehicle is detected in the image taken by camera then green signal is turned on for the emergency vehicle.

Use Case Diagram of SmartFlow:

Capture image

Traffic Signal

Vehicle detection

Congestion control

Image processing

Camera Module

AI

Traffic Management

Emergency Vehicle Detector

Figure 3: Use case Diagram

The figure 3 represents the use cases and actors in the SmartFlow system which can be described as follows

Use Cases

- Traffic signal: The traffic junction where the traffic lights are controlled by the traffic management system.

- Capture image: The cameras at junction takes images and send it to the system.

- Image processing: The AI algorithm processes the image to detect the number of vehicles and classify them.

- Vehicle detection: The AI algorithm uses data set and detect type and number of vehicles and detect the emergency vehicle.

- Congestion control: Traffic congestion, were there are more vehicles in one lane and a smaller number of vehicles in another lane leads to traffic.

Actors

- Camera Module: Captures the real time image or video to control the congestion based on the number of vehicles on a lane.

- AI: Artificial intelligence algorithm YOLO used to detect the vehicles and classify them to calculate the green delay.

- Traffic Management: The main system which controls the normal cycle of traffic and congestion control of the traffic, it uses the data given by the AI algorithm control the traffic by adjusting green delay.

- Emergency Vehicle Detector: The text detection algorithm is used to detect the emergency vehicle and inform system to turn on the green signal for emergency vehicle.

State Chart Diagram of SmartFlow:

Real-Time Traffic

Capture Image

Vehicle Detection and Count Vehicles

Calculate Green Delay

Update the Signal

Ambulance Detection

Store In Database

Figure 4: State Chart Diagram

As shown in the figure 4 the process starts with the system in an idle state, waiting to capture image in last 5 seconds of green delay. When the camera captures image of a lane, that image is stored in the database. The AI algorithm then retrieves these stored images to process them and count the number of vehicles in the lane. Based on the vehicle count, the system calculates the duration for the green signal in a lane. As the green signal period for a lane is about to end, the camera captures a new image of the next lane to prepare for the subsequent cycle.

Simultaneously, the AI algorithm continuously analyses the images to detect any emergency vehicles, such as ambulances. If an emergency vehicle is detected, the system immediately overrides the normal operation and activates the green signal for the lane with the emergency vehicle, allowing it to pass through without delay. After the emergency vehicle has passed, the system returns to its idle state, ready to capture the next set of images at the fixed interval, ensuring efficient traffic management and prioritization of emergency vehicles.

Sequence Diagram

Update green signal

Capture image Save image to Database

Retrieve Image and count no of vehicles

Capture image Save image to Database

Detect Emergency vehicle

If emergency Update green signal

ESP-32

Data Base

AI

Traffic signal

Figure 5: Sequence Diagram

As shown in the figure 5 camera captures the image and store it in the database, from database image is retrieved by AI algorithm to count the number od vehicles and calculate the green delay, this continues for all other lanes, when last few seconds of green delay camera takes picture of other lane. But the camera captures images of all lanes at fixed time of interval and that images are taken by the AI algorithm to detect the emergency vehicle this is continuous process so that emergency vehicles like ambulance will not suffer from traffic when the ambulance is detected the green signal is turned on.

Module 1: Congestion control

Load Vehicle detector

Load YOLO Weights

Read COCO dataset

Return Green Delay

Draw Boxes

Count the vehicles

Calculate Green Delay

Read Image

Detect the vehicles

Figure 6: Congestion control

The above figure 6 represents the block diagram of module one congestion control. As shown in the figure Vehicle detector algorithm is loaded which uses YOLO weights and coco dataset to detect the vehicles in the image which was extracted from the database. Then using cv2.dn DetectionModel() vehicles are detected and boxes are drawn around the vehicles and number of boxes are returned. The number of boxes represents the total number of vehicles in the lane using that green delay is calculated and returned.

As shown in the figure 6 for congestion control, the Vehicle Detector Algorithm plays a pivotal role in managing traffic flow efficiently. This algorithm, integrated with YOLO weights and the COCO dataset, utilizes deep learning capabilities to detect vehicles within images extracted from the database. Employing OpenCV's cv2.dnn DetectionModel(), the algorithm performs vehicle detection by drawing bounding boxes around identified vehicles and counting these boxes to determine the total number of vehicles in the lane. This count forms the basis for calculating the green delay required for the traffic signal, considering factors such as average vehicle speed. The calculated green delay value is crucial for dynamically adjusting traffic signals via systems like the Arduino Mega, ensuring optimal traffic flow management in real-time scenarios. This process underscores how advanced image processing and machine learning techniques enable the SmartFlow system to respond dynamically to traffic conditions, thereby enhancing road efficiency and safety.

Module 2: Emergency Vehicle Detection

Return True

Else Return False

If Ambulance

Read Image

Resize the Image

Set path to OCR

Text Detection and Draw boxes

Load text detection model

Recognize Text Within ROI

Figure 7: Emergency Vehicle Detection

As shown in the figure 7 the emergency vehicle detection model works as follows, it uses the text detection model frozen_east_text_detection.pb which is pre trained model for text detection. When image is extracted from the database is resized as required by the model and it is passed into model for text detection when it detected the text draw the boxes around it then Text Recognition (OCR) Extract the region of interest (ROI) from the original frame. That detected text is stored in a variable to check whether it is AMBULANCE or not. If it is ambulance return true else returns false.

Emergency vehicle

ESP32-Cam

Arduino uno

Database

Arduino Mega

Traffic Signal

System

Real Time Traffic

Capture image

Program the ESP32

Save to Database

Emergency vehicle

Detection

Capture image/ video

Load to Arduino

Updating the signal

Load image